FIRELINE

Data Exploration & Findings Report

NASA FIRMS • Pontianak Weather • Forest Fire

# 1. Executive Summary

## NASA FIRMS

61,583 hotspot detections from 1 Aug 2024 to 31 May 2026.

0 missing values, 0 exact duplicates, and 0 spatial-temporal duplicates under (latitude, longitude, date, time).

July–September contributes 71.9% of hotspots and 78.7% of total FRP.

FRP is highly right-skewed: median 6.14 MW, 99th percentile 70.20 MW, maximum 954.79 MW.

Confidence is 93.1% nominal, 3.8% high, 3.2% low; reliability must remain separate from severity.

## Pontianak Weather

1,734 raw rows become 1,460 unique dates after duplicate-date cleanup.

Rainfall RR is missing in 26.2% of dates; TAVG and RH_AVG are nearly complete.

Temperature and humidity have a strong inverse relationship (Pearson about -0.816).

Single-station coverage means this is not automatically representative of all Kalimantan.

## Forest Fire Dataset

517 rows, no missing values, 4 exact duplicates.

Dataset is Montesinho Natural Park, Portugal, 2000–2003.

Use as methodological/feature reference, not as Kalimantan historical evidence.

# 2. NASA FIRMS Variable Reference

| Variable | Meaning | Unit/code | FIRELINE role | Caution |
| --- | --- | --- | --- | --- |
| latitude | Hotspot latitude | degrees | Spatial join / map | Needs admin boundary join |
| longitude | Hotspot longitude | degrees | Spatial join / map | Same |
| acq_date | Satellite acquisition date | date | Temporal joins | Not necessarily ignition time |
| acq_time | Acquisition time | HHMM UTC | Sub-daily patterns | Keep UTC consistent |
| brightness | Fire-pixel brightness temperature | K | Thermal feature | Nonlinear/saturation effects |
| bright_t31 | Thermal/background reference field | K | Thermal context | Verify exact field semantics for product export |
| frp | Fire Radiative Power | MW | Intensity / urgency | Proxy, not full risk |
| confidence | Detection confidence | l/n/h | Reliability / verification | Not severity |
| daynight | Day/night flag | D/N | Observation context | Not universal accuracy rank |
| scan | Across-scan geometry metric | product-specific | QC/geometry | Can affect comparisons |
| track | Along-track geometry metric | product-specific | QC/geometry | Same |
| type | Detection type | code | Product context | Constant 0 here |
| version | Product version | version | Reproducibility | Not risk predictor |

# 3. NASA FIRMS Findings

## Finding 1 — Strong temporal concentration

Evidence: July–September represents 71.9% of hotspots and 78.7% of total FRP. The 2024/2026 periods are partial, so cross-year comparisons need coverage normalization.

Action: Build seasonal baselines, daily load indicators, and rolling windows; avoid fixed month-only rules.

## Finding 2 — FRP is useful but heavy-tailed

Evidence: Median 6.14 MW, P90 22.11 MW, P99 70.20 MW, max 954.79 MW.

Action: Use percentile/log transforms and combine intensity with exposure/environment.

## Finding 3 — Confidence ≠ severity

Evidence: High/nominal/low are reliability classes. Low-confidence high-FRP cases exist in the supplied extract.

Action: Keep confidence separate; use it for evidence strength/verification rather than deleting detections.

## Finding 4 — Spatial clustering is real but admin context is missing

Evidence: Coordinate grid cells show concentrated hotspot zones, but province/district names require an explicit spatial join.

Action: Join administrative boundaries, land, settlements and facilities before assigning operational labels.

## Finding 5 — Thermal anomalies need QC flags

Evidence: 57 rows have brightness 207.93 K; 166 rows have negative brightness−bright_t31, including high-FRP cases.

Action: Preserve raw records, flag for review, and verify product semantics before filtering.

NASA daily hotspots and 7-day rolling mean

NASA FRP by confidence

NASA monthly hotspot pattern by year

NASA spatial FRP pattern

# 4. Pontianak Weather Findings

| Variable | Observed summary | Potential relevance | Limitation |
| --- | --- | --- | --- |
| TAVG | Mean 27.68°C; 24–31°C | Hot/dry context; candidate feature | Station-specific |
| RH_AVG | Mean 82.64%; 65–96% | Lower humidity may indicate drier conditions | Station-specific |
| RR | Median 5.5 mm; 26.2% missing | Rainfall/moisture context | High missingness |
| date | 2021–2024 daily | Temporal alignment | Overlap with NASA is limited |

Key action: do not blindly impute all rainfall. Consider missingness flags and use broader station/grid weather for production Kalimantan coverage.

# 5. Forest Fire Dataset Findings

This dataset is from Montesinho Natural Park, Portugal, 2000–2003. It should not be merged as empirical Kalimantan history. Its value for FIRELINE is methodological: it exposes a useful vocabulary of fire-weather variables and FWI components.

| Variable | Meaning | FIRELINE use |
| --- | --- | --- |
| X, Y | Local 9×9 spatial grid | Demonstrates spatial fire context; not reusable as Kalimantan coordinates |
| month, day | Calendar context | Temporal feature ideas |
| FFMC | Fine Fuel Moisture Code | Fine-fuel dryness concept |
| DMC | Duff Moisture Code | Surface organic-layer dryness concept |
| DC | Drought Code | Longer-term drought/drying concept |
| ISI | Initial Spread Index | Spread-potential concept |
| temp | Temperature | Environmental context |
| RH | Relative humidity | Environmental context |
| wind | Wind speed | Potential spread context |
| rain | Rainfall | Moisture context |
| area | Burned area (ha) | Historical outcome in Portugal only |

# 6. Integration Blueprint

Use NASA FIRMS as the current event/hotspot backbone. Enrich it using spatial-temporal joins to weather, population/schools, land cover, historical recurrence, and later operational access.

| Dataset | Spatial compatibility | Temporal compatibility | Recommendation |
| --- | --- | --- | --- |
| NASA FIRMS | Native point coordinates | Current event period | Core event table |
| Pontianak Weather | Single station | 144 dates overlap with NASA extract | Prototype local weather context; not all-Kalimantan |
| Forest Fire | Portugal 9×9 grid | 2000–2003 | Method reference only; find Kalimantan history for production |
| Population/School | Needs spatial join | Annual/static | Exposure layer |
| Peatland/Land cover | Needs spatial join | Static/periodic | Hazard/vulnerability context |

# 7. Feature Engineering Recommendations

FRP percentile / log(FRP)

Daily hotspot count

Daily total FRP

7-day rolling hotspot count and total FRP

Historical hotspot frequency by spatial cell

Confidence reliability feature

Population exposure within a buffer

Distance/count of schools and critical facilities

Rainfall accumulation / dry-day sequence once appropriate weather coverage is available

Do not yet create a universal “critical FRP = critical risk” rule, infer province names from longitude, or train a production FIRELINE model on the Portugal Forest Fire dataset.

# 8. Immediate DS Next Steps

Standardize NASA event fields and create QC flags.

Build a date + spatial weather join prototype using Pontianak as a local case, not a Kalimantan-wide assumption.

Find geographically relevant historical fire data to replace Portugal data for empirical modeling.

Add population and school exposure through spatial joins.

Test an interpretable baseline scoring/index before a more complex ML model.

# 9. Final Finding for FIRELINE

FIRELINE should not be framed as a hotspot detector. NASA FIRMS already provides the current fire-signal backbone. The differentiator is context enrichment: environmental conditions, historical recurrence, human exposure, critical facilities, and eventually operational accessibility. This turns fragmented data into a decision-support queue for the Command Center.

References: NASA Earthdata, Collection 2 VIIRS 375 m Active Fire User Guide; NASA Earthdata FIRMS FAQ / Forum; UCI Machine Learning Repository, Forest Fires dataset (Montesinho Natural Park, Portugal, 2000–2003).